OSCN LAB

ASSIGNMENT-4

Name: Anjali Kiran G

Roll. No.: 2503B05102

M.TECH CSE (semester -1)

Question:

Write a C++ program to implement Hamming Code for error detection and correction.

Requirements:

- Accept a binary data string as input.

- Determine the number of redundant bits required.

- Insert the redundant bits into appropriate positions (1, 2, 4, 8, …).

- Calculate and set the parity bits using even parity logic.

- Display the final data packet (data bits + redundant bits) to be transmitted.

- Simulate the receiver side to:

  - Recalculate parity bits.

  - Detect if there is any single-bit error.

  - Display the position of the erroneous bit or confirm no error if data is correct.

Code:

#include <iostream>

#include <cmath>

#include <vector>

using namespace std;

int getRedundantBits(int m) {

int r = 0;

while (pow(2, r) < (m + r + 1)) r++;

return r;

}

vector<int> generateHamming(vector<int> data) {

int m = data.size();

int r = getRedundantBits(m);

int total = m + r;

vector<int> hamming(total + 1);

int j = 0;

for (int i = 1; i <= total; i++) {

if ((i & (i - 1)) == 0) {

hamming[i] = 0;

} else {

hamming[i] = data[j++];

}

}

for (int i = 0; i < r; i++) {

int pos = pow(2, i);

int parity = 0;

for (int k = 1; k <= total; k++) {

if (k & pos) parity ^= hamming[k];

}

hamming[pos] = parity;

}

return hamming;

}

int detectError(vector<int> code) {

int total = code.size() - 1;

int r = 0;

while (pow(2, r) <= total) r++;

int errorPos = 0;

for (int i = 0; i < r; i++) {

int pos = pow(2, i);

int parity = 0;

for (int k = 1; k <= total; k++) {

if (k & pos) parity ^= code[k];

}

if (parity != 0) errorPos += pos;

}

return errorPos;

}

int main() {

string input;

cout << "Enter original binary data: ";

cin >> input;

vector<int> data;

// FIXED: works on all compilers

for (int i = 0; i < input.length(); i++) {

data.push_back(input[i] - '0');

}

vector<int> hamming = generateHamming(data);

cout << "\nGenerated Hamming Code: ";

for (int i = 1; i < hamming.size(); i++)

cout << hamming[i];

cout << endl;

string received;

cout << "\nEnter received Hamming Code: ";

cin >> received;

vector<int> rec(received.size() + 1);

for (int i = 1; i <= received.size(); i++)

rec[i] = received[i - 1] - '0';

int errorPos = detectError(rec);

if (errorPos == 0) {

cout << "\nNo error detected.\n";

} else {

cout << "\nError detected at bit position: " << errorPos << endl;

rec[errorPos] ^= 1;

cout << "Corrected Code: ";

for (int i = 1; i < rec.size(); i++)

cout << rec[i];

cout << endl;

}

return 0;

}

Output:

Hamming Code Explanation:

- The data bits are taken as input and the number of redundant (parity) bits required is calculated using the condition
2^r ≥ m + r + 1.

- The redundant bits are placed at positions that are powers of 2, such as 1, 2, 4, 8, etc.

- Each parity bit is calculated using even parity, meaning the total number of 1s in its covered positions must be even.

- The final Hamming code is formed by combining data bits and parity bits and is transmitted to the receiver.

- At the receiver side, parity bits are recalculated; if their combination gives a non-zero value, that position indicates the bit where the error occurred, and the program corrects it.